AI SMART GRID

Energy Monitoring System

Technical Documentation — LSTM AI Model & System Architecture

Developed by: Prathamesh Sapate

Date: March 2026   |   Version: 4.0

# Table of Contents

1.  System Overview

2.  Hardware Components

3.  ESP32 Firmware — How Sensors Work

3.1  ACS712 Current Sensor

3.2  ZMPT101B Voltage Sensor

3.3  DHT11 Temperature & Humidity Sensor

4.  Backend Server (Flask + Python)

5.  LSTM AI Model — How It Works

5.1  What is LSTM?

5.2  Model Architecture

5.3  Input Features

5.4  Training Process

5.5  Prediction Flow (Step by Step)

5.6  MinMaxScaler

6.  Data Flow — End to End

7.  Dashboard & Visualization

8.  API Endpoints Reference

9.  Database Schema

10.  Calibration Guide

11.  Troubleshooting

# 1. System Overview

──────────────────────────────────────────────────────────────────────────────────────────

AI Smart Grid is an intelligent energy monitoring system that uses an ESP32 microcontroller to read real-time sensor data (voltage, current, temperature, humidity) and sends it to a Python Flask server. The server uses a trained LSTM (Long Short-Term Memory) neural network to predict future power consumption. A live web dashboard displays all data, charts, AI predictions, alerts, and historical trends.

## System Block Diagram

┌─────────────────────────────────────────────────────────────────────┐
  │                        AI SMART GRID SYSTEM                         │
  └─────────────────────────────────────────────────────────────────────┘

  ┌──────────────┐      WiFi/HTTP       ┌──────────────────────────────┐
  │    ESP32     │ ──── POST JSON ────► │   Flask Server (Python)      │
  │              │                      │   • Receives sensor data      │
  │  ACS712      │                      │   • Runs LSTM prediction      │
  │  ZMPT101B    │                      │   • Stores in SQLite DB       │
  │  DHT11       │                      │   • Serves REST APIs          │
  └──────────────┘                      └──────────────┬───────────────┘
                                                        │
                                                        ▼
                                         ┌─────────────────────────┐
                                         │   Web Dashboard         │
                                         │   • Live Charts         │
                                         │   • AI Predictions      │
                                         │   • Historical Data     │
                                         │   • Energy Forecast     │
                                         └─────────────────────────┘

# 2. Hardware Components

──────────────────────────────────────────────────────────────────────────────────────────

| Component | Model | Pin (ESP32) | Purpose |
| --- | --- | --- | --- |
| Microcontroller | ESP32 (30-pin) | — | Main controller, WiFi, ADC |
| Current Sensor | ACS712-20A | GPIO 34 (Analog) | Measures AC current (0–20A) |
| Voltage Sensor | ZMPT101B | GPIO 35 (Analog) | Measures AC voltage (0–250V) |
| Temp & Humidity | DHT11 | GPIO 4 (Digital) | Room temperature & humidity |
| Power Supply | 5V USB / 3.3V | 3.3V & GND | Powers ESP32 & sensors |

# 3. ESP32 Firmware — How Sensors Work

──────────────────────────────────────────────────────────────────────────────────────────

## 3.1  ACS712 Current Sensor (AC Current)

The ACS712-20A sensor works on the Hall Effect principle. When AC current flows through the sensor's wire, it generates a proportional voltage on the output pin.

📐 Method Used:

Peak-to-Peak (Vpp) Method — samples ADC for 500ms, finds maximum and minimum values

Sensitivity: 100 mV/A (for ACS712-20A module)

Formula:

Step 1: Sample ADC for 500ms → find maxValue and minValue
  Step 2: Vpp = ((maxValue - minValue) × 4.9) / 4095
  Step 3: VRMS = (Vpp / 2) × 0.707          ← Vpp to RMS conversion
  Step 4: AmpsRMS = (VRMS × 1000 / 100) - 0.3  ← 100 mV/A, 0.3A offset
  Step 5: if AmpsRMS < 0 → set to 0

  Auto-Calibration at Startup:
  → Measures baseline Vpp with no load (ADC noise)
  → Subtracts it from every future reading
  → Prevents false 2-3A readings from ADC noise

## 3.2  ZMPT101B Voltage Sensor (AC Voltage)

The ZMPT101B is a precision voltage transformer that steps down 230V AC to a safe level readable by the ESP32 ADC (0-3.3V).

📐 Method Used:

Peak Detection Method — samples 100 ADC readings with 1ms delay each

Only values above ADC threshold 3300 are counted (positive half-cycle peaks only)

Formula:

Step 1: Read 100 ADC samples with 1ms delay
  Step 2: val[i] = (ADC > 3300) ? ADC_value : 0  ← filter noise
  Step 3: Find max_v = maximum of all val[]
  Step 4: VmaxD = max_v
  Step 5: VeffD = VmaxD / √2                ← Peak to RMS
  Step 6: Veff  = (VeffD / 100.24) × 15     ← Calibration formula
  Step 7: Veff  = Veff × VOLTAGE_SCALE       ← Fine-tune (default 0.630)
  Step 8: if max_v == 0 → Veff = 0 (no AC detected)

## 3.3  DHT11 Temperature & Humidity Sensor

The DHT11 uses a single-wire digital protocol. The ESP32 reads temperature (°C) and relative humidity (%) using the DHT library. If reading fails (isnan check), fallback values 25°C / 50% are used to prevent NaN in the JSON payload.

## 3.4  Data Transmission (WiFi HTTP POST)

Every 5 seconds, the ESP32 sends a JSON payload to the Flask server:

POST http://192.168.x.x:5050/api/sensor
  Content-Type: application/json

  {
    "voltage":     230.6,
    "current":     0.20,
    "temperature": 32.4,
    "humidity":    35.0,
    "power":       46.12
  }

Server discovery: mDNS (smart-grid.local) with fallback to hardcoded IP.

# 4. Backend Server (Flask + Python)

──────────────────────────────────────────────────────────────────────────────────────────

The Flask server (app.py) is the brain of the system. It:

Receives sensor data from ESP32 via HTTP POST

Stores every reading in a SQLite database (database.py)

Maintains a rolling buffer of 12 recent readings for LSTM input

Runs LSTM prediction after every new reading (when buffer is full)

Serves the web dashboard (index.html) and all REST API endpoints

Broadcasts itself on the network using mDNS (Zeroconf)

## Key Components:

| File | Purpose |
| --- | --- |
| app.py | Main Flask server — routes, LSTM inference, demand response logic |
| database.py | SQLite DB — store/retrieve sensor readings, cleanup thread |
| lstm_model.keras | Trained LSTM neural network (363 KB) |
| scaler.pkl | MinMaxScaler — normalizes input data for LSTM |
| templates/index.html | Single-page web dashboard (HTML + JS + Chart.js) |
| train_model.py | Script to train/retrain the LSTM model from DB data |

# 5. LSTM AI Model — How It Works

──────────────────────────────────────────────────────────────────────────────────────────

## 5.1  What is LSTM?

LSTM stands for Long Short-Term Memory. It is a special type of Recurrent Neural Network (RNN) that can remember patterns over long sequences of data. For energy monitoring, this means the model learns: "If voltage is high + temperature is high + humidity is low in the past 12 readings, then current (power consumption) will likely be X amperes in the next reading."

❓ Why LSTM for energy prediction?

Energy consumption follows TIME PATTERNS — morning, afternoon, evening usage differs

LSTM has "memory cells" that remember past values and their effect on future values

Unlike simple regression, LSTM understands sequences — "what happened before matters"

Works well with only 3 input features (voltage, temperature, humidity)

## 5.2  Model Architecture

Input: (batch_size, 12, 3)  ← 12 time steps, 3 features each
         [Voltage(V), Humidity(%), Temperature(°C)]

  Layer 1: LSTM(50 units, return_sequences=True)
           → Output: (batch, 12, 50)  [remembers all 12 steps]

  Layer 2: Dropout(0.2)  ← prevents overfitting (drops 20% neurons randomly)

  Layer 3: LSTM(50 units, return_sequences=False)
           → Output: (batch, 50)  [compresses to 50 features]

  Layer 4: Dropout(0.2)  ← prevents overfitting

  Layer 5: Dense(1)  ← single output neuron
           → Output: (batch, 1)  [predicted_current in Amps]

  Total Parameters: 93,155
  Trained Params:   31,051
  Optimizer: Adam  |  Loss: Mean Squared Error (MSE)

## 5.3  Input Features

| Feature | Unit | Why It Matters | Typical Range |
| --- | --- | --- | --- |
| Voltage | V | Higher voltage → more power drawn by loads | 215 – 245 V |
| Humidity | % | High humidity → fans, ACs, dehumidifiers ON | 20 – 90 % |
| Temperature | °C | Hot day → AC ON, Cold day → Heater ON | 10 – 45 °C |
| Current (target) | A | What the model predicts (output) | 0 – 20 A |

## 5.4  Training Process

The model is trained using historical sensor data stored in the SQLite database:

1. Load all readings from readings table (voltage, humidity, temperature, current)
  2. Scale all values to range [0, 1] using MinMaxScaler
  3. Create sequences of length 12:
     X = [[v1,h1,t1], [v2,h2,t2], ..., [v12,h12,t12]]  ← 12 timesteps
     y = current at timestep 13  ← what to predict
  4. Split: 80% training, 20% validation
  5. Train LSTM for 10 epochs with batch_size=32
  6. Save model → lstm_model.keras
  7. Save scaler → scaler.pkl  (needed to un-scale predictions)

🔄 To retrain the model with new data:

python3 train_model.py

## 5.5  Prediction Flow — Step by Step

EVERY TIME ESP32 SENDS DATA:

  Step 1: Server receives JSON → extracts voltage, humidity, temperature, current

  Step 2: Append to rolling buffer (deque, maxlen=12)
          recent_history = [...last 12 readings...]

  Step 3: If buffer NOT full (len < 12):
          → status = "Collecting Sequence... (7/12)"
          → prediction = 0.0 (not ready yet)

  Step 4: If buffer IS full (len == 12):
          a. Build input matrix X (12 × 3):
             [[v1,h1,t1], [v2,h2,t2], ..., [v12,h12,t12]]

          b. Scale each row using MinMaxScaler:
             scaled = scaler.transform([[v, h, t, 0]])[0][:3]

          c. Reshape for LSTM: X = [[scaled_sequence]] → shape (1, 12, 3)

          d. Run LSTM: predicted_scaled = model.predict(X)

          e. Inverse scale the prediction:
             dummy = [0, 0, 0, predicted_scaled]
             predicted_current = scaler.inverse_transform(dummy)[3]

          f. Clamp to 0 if negative: predicted_current = max(0, predicted_current)

  Step 5: Check demand response:
          if predicted_current > 3.0A → "⚠️ WARNING: Heavy Load Predicted"
          if predicted > actual × 1.5 → "⚡ Notice: Load Spike Imminent"
          else                        → "✅ System Normal"

  Step 6: Store in database, update latest_data dict
  Step 7: Return response to ESP32: {"prediction": 2.94, "power": 46.12}

## 5.6  MinMaxScaler — Why We Need It

Neural networks work best when all input values are in the same scale (0 to 1). Without scaling, voltage (230V) would dominate over humidity (35%) and the model would give wrong predictions.

Without scaling:  voltage=230,  humidity=35,  temperature=32
  With MinMaxScaler: voltage=0.75, humidity=0.18, temperature=0.41

  Formula: scaled = (x - min) / (max - min)

  The scaler was fitted on training data and saved as scaler.pkl
  Same scaler MUST be used at prediction time (loaded from scaler.pkl)

# 6. Data Flow — End to End

──────────────────────────────────────────────────────────────────────────────────────────

[PHYSICAL WORLD]
      AC Mains (230V, current load)
           │
           ▼
  [ESP32 SENSORS] — every 5 seconds
      ACS712  → reads AC current (A)
      ZMPT101B → reads AC voltage (V)
      DHT11   → reads temperature (°C) & humidity (%)
           │
           ▼  HTTP POST JSON
  [FLASK SERVER — app.py]
      1. Parse JSON
      2. Append to rolling buffer (deque 12)
      3. LSTM predict → predicted_current
      4. Check demand response threshold
      5. Write to SQLite DB
      6. Update latest_data{} dict
      7. Return {prediction, power} to ESP32
           │
           ▼  HTTP GET (every 2 seconds)
  [DASHBOARD — index.html]
      fetchLive()    → /api/data          → shows live values
      fetchHistory() → /api/history       → shows charts
      fetchForecast()→ /api/energy-forecast → shows kWh/bill
      fetchSmart()   → /api/smart-forecast → 24h AI forecast
      fetchAlerts()  → /api/alerts         → demand response log

# 7. Dashboard & Visualization

──────────────────────────────────────────────────────────────────────────────────────────

| Page | What It Shows | Update Rate |
| --- | --- | --- |
| Dashboard | Live voltage, current, temp, humidity, power, AI prediction, forecast strip | Every 2s |
| AI Insights | LSTM model info, actual vs predicted chart, prediction error bar chart | Every 2s |
| Energy Analytics | Real-time power chart, hourly energy breakdown, actual vs predicted power | Every 2s |
| Historical Data | Combined chart + individual charts for all sensors, time range filters | Every 30s |

# 8. API Endpoints Reference

──────────────────────────────────────────────────────────────────────────────────────────

| Method | Endpoint | Used By | Returns |
| --- | --- | --- | --- |
| POST | /api/sensor | ESP32 | prediction, power — stores new reading |
| GET | /api/data | Dashboard | Latest snapshot: voltage, current, prediction, status |
| GET | /api/history?range=1h | Dashboard | List of DB readings for given time range |
| GET | /api/energy-forecast | Dashboard | kWh consumed today/30d, AI predicted kWh/bill |
| GET | /api/smart-forecast | Dashboard | 24-hour AI forecast with weather-based multipliers |
| GET | /api/alerts | Dashboard | Demand response alert log |
| GET | /api/ai-info | Dashboard | Model architecture, buffer status, total readings |
| GET | / | Browser | Serves the main dashboard HTML |

# 9. Database Schema (SQLite)

──────────────────────────────────────────────────────────────────────────────────────────

Table: readings

  Column             Type     Description
  ─────────────────────────────────────────────────────────
  id                 INTEGER  Primary key, auto-increment
  timestamp          TEXT     Reading time (YYYY-MM-DD HH:MM:SS)
  voltage            REAL     AC voltage in Volts
  current_val        REAL     AC current in Amperes
  temperature        REAL     Temperature in °C
  humidity           REAL     Relative humidity in %
  predicted_current  REAL     LSTM predicted current in A
  power              REAL     Calculated power (V × I) in Watts
  relay_state        TEXT     Relay status (N/A for this version)
  ─────────────────────────────────────────────────────────
  Retention: 15 days  (auto-cleanup thread deletes older rows)
  Location: energy_data.db (SQLite, no installation needed)

# 10. Calibration Guide

──────────────────────────────────────────────────────────────────────────────────────────

## Current Sensor Calibration (ACS712-20A)

The ESP32 auto-calibrates at startup with no load connected

Serial Monitor shows: [CAL] Zero-load Vpp baseline = 0.7680 V

If reading shows small current at 0A load: restart ESP32 with NO load connected

mVperAmp = 100 (for 20A module), 185 (for 5A module), 66 (for 30A module)

File: esp32_code.ino
  Line: int mVperAmp = 100;  // change for your module version

## Voltage Sensor Calibration (ZMPT101B)

VOLTAGE_SCALE is the calibration factor. Default: 0.630

Measure actual voltage with a multimeter, then adjust:

VOLTAGE_SCALE = actual_voltage / measured_voltage
  Example: 220.0 / 349.3 = 0.630

  File: esp32_code.ino
  Line: #define VOLTAGE_SCALE 0.630f  // ← tune this

# 11. Troubleshooting

──────────────────────────────────────────────────────────────────────────────────────────

| Problem | Cause | Solution |
| --- | --- | --- |
| Current shows 2-3A at 0A load | ESP32 ADC noise | Restart ESP32 with no load for auto-calibration |
| Voltage shows 300-350V | VOLTAGE_SCALE not set | Set VOLTAGE_SCALE = actual_V / measured_V |
| HTTP -1 error | Wrong server IP | Update SERVER_IP in esp32_code.ino to your PC's IP |
| mDNS not found | Router blocks multicast | Fallback to SERVER_IP works automatically |
| AI Predicted shows -- | Buffer not full (< 12 readings) | Wait ~60 seconds for 12 readings to accumulate |
| Avg voltage shows 50V | 0V readings included in avg | Fixed in dashboard — only non-zero readings averaged |
| Model not loaded | lstm_model.keras missing | Run: python3 train_model.py |
| Flask not starting | TensorFlow import error | Run: pip3 install tensorflow |
| DHT reads NaN | Loose wire or poor contact | Check DHT11 wiring on GPIO4, use 10kΩ pull-up resistor |

## How to Start the System

1. Start Flask server:
     cd /path/to/smart_energy_project_clean
     python3 app.py

  2. Open dashboard in browser:
     http://127.0.0.1:5050
     or http://192.168.x.x:5050  (from other devices)

  3. Upload ESP32 firmware:
     Open: ESP code/esp32_code/esp32_code.ino in Arduino IDE
     Select: Board → ESP32 Dev Module
     Click: Upload

  4. Open Serial Monitor (115200 baud) to see:
     [CAL] Calibrating current sensor...
     [WiFi] Connected! IP: 192.168.x.x
     [mDNS] Server found: http://192.168.x.x:5050/api/sensor
     [SENSOR] V=230.5V  I=0.20A  T=32.4°C  H=35.0%  P=46.1W
     [TX] {...} → HTTP 200

──────────────────────────────────────────────────────────────────────────────────────────

AI Smart Grid — Technical Documentation | Prathamesh Sapate | March 2026